Mittag-Leffler Stability Analysis for a fractional order model of co-contaminated soil and water pollution

Priya P.1, Roselyn Besi P.1 Revathy M1, Ali Akgul2,3,*

1Department of Mathematics, Coimbatore Institute of Technology,

Coimbatore, Tamil Nadu, India

2Department of Electronics and Communication Engineering, Saveetha School of Engineering,SIMATS, Chennai, Tamilnadu, India

3Siirt University, Art and Science Faculty, Department of Mathematics, 56100 Siirt, Türkiye

Corresponding author: priyabarath612@gmail.com1*, aliakgul@siirt.edu.tr

Abstract

Environmental pollution is a serious and long-lasting problem, and understanding how it spreads and how it can be controlled requires robust mathematical models. In this work, we study a five-compartment fractional-order model that describes how pollution moves between different environmental components, namely effluent, soil, water, and farmland. To capture the memory and hereditary effects that naturally occur in environmental processes, we use the Atangana–Baleanu–Caputo (ABC) fractional derivative, which provides a more realistic description than classical integer-order models. We first analyze the qualitative behavior of the model. Using linearization, Lyapunov function, and tools from fractional calculus, we establish the local and global asymptotic stability of two important equilibrium points: the pollution-free equilibrium (E₀) and the endemic (polluted) equilibrium (E*). In particular, we apply the concept of Mittag-Leffler stability, which is well-suited to fractional-order systems. A central result of the analysis is the derivation of the basic reproduction number

R0= LaNΦC Φ2+ γ1,

which acts as a threshold parameter for pollution dynamics. If R₀ < 1, the pollution-free equilibrium E₀ is globally asymptotically stable, meaning that pollution will eventually disappear from the environment. On the other hand, if R₀ > 1, the pollution-free state becomes unstable, and a unique endemic equilibrium E* exists and is stable, indicating that pollution will persist in the long term. To support the theoretical results, we develop an adaptive predictor–corrector numerical scheme designed specifically for the ABC fractional-order model. This method is both accurate and computationally efficient, making it suitable for long-time simulations. Numerical experiments, particularly for the case R₀ = 4.55, clearly show that the solution converges to the endemic equilibrium, which confirms the theoretical stability results and demonstrates the reliability of the proposed numerical approach. Overall, this study provides a clear mathematical framework for understanding pollution spread and control in complex environmental systems. The analytical results, together with the numerical simulations, offer valuable insights that can help in designing effective long-term pollution remediation and management strategies.

Keywords: Co-Contamination, ABC mathematical model, fixed point, Adaptive control, Adams-Bhashforth.

1. Introduction

Environmental pollution affecting soil and water systems has emerged as a serious global challenge due to industrial discharge, agricultural runoff, and uncontrolled waste disposal. Since soil and water environments are strongly coupled, contaminants introduced into one medium can easily migrate to the other, leading to persistent ecological damage and long-term health risks. Mathematical modeling is an essential tool for understanding such complex pollution dynamics and for designing effective remediation strategies. However, classical integer-order models often fail to capture the memory effects, nonlocal interactions, and anomalous transport mechanisms observed in real environmental processes. Fractional calculus provides a powerful and flexible framework for modeling systems with memory and hereditary properties. Fractional-order differential equations have been successfully applied in various scientific fields, including heat transfer, diffusion, epidemiology, groundwater flow, and ecological systems (Diethelm, 2010; Baleanu et al., 2019). Unlike integer-order models, fractional models are capable of representing long-range temporal dependence and anomalous diffusion. In recent years, special attention has been given to fractional derivatives with nonlocal and non-singular kernels. Among them, the Atangana–Baleanu fractional derivative in Caputo sense has attracted considerable interest due to its Mittag-Leffler kernel, which avoids singularities and provides better physical interpretation (Atangana & Baleanu, 2017). This operator has been widely applied to heat transfer models and groundwater flow systems (Atangana & Algahtani, 2015; Sun et al., 2019). Efficient numerical methods for solving fractional-order systems with the Atangana–Baleanu–Caputo derivative have been actively developed, showing good stability and convergence properties (Yadav & Pandey, 2019; Singh et al., 2022; Yadav & Pandey, 2020). Stability analysis is fundamental in fractional-order dynamical systems, where stability is governed by Mittag-Leffler functions rather than exponential decay. Rigorous stability criteria and Lyapunov-based methods have been developed for such systems (Li & Zhang, 2019; Vargas-De-León, 2012). Motivated by these developments, this study proposes a fractal fractional-order model for co-contaminated soil and water pollution using the Atangana–Baleanu–Caputo derivative. The local and global stability of the system equilibria is analyzed using Mittag-Leffler stability theory, supported by numerical simulations.

This study contributes to the field by presenting a detailed and rigorous analysis of a five-compartment fractional-order model for pollution dynamics and remediation, specifically utilizing the Atangana-Baleanu-Caputo derivative. We extend previous work by providing a thorough local and global stability analysis of both the pollution-free (trivial) and persistent (endemic) equilibrium points. Our analysis employs advanced techniques such as Mittag-Leffler stability and fractional Lyapunov methods to characterize the system’s long-term behavior precisely. Furthermore, we develop and implement an advanced numerical scheme tailored for the efficient and accurate simulation of the ABC fractional system, enabling practical exploration of model behavior and validation of the theoretical findings. This comprehensive approach, combining rigorous theoretical analysis with robust computational tools, aims to provide deeper insights into the conditions necessary for successful pollution remediation and the long-term behavior of complex, memory-dependent environmental systems.

2. Mathematical Preliminaries

2.1 Fractional Calculus Fundamentals

Definition 2.1: (Atangana-Baleanu-Caputo Fractional Derivative). [1]  For 0<α<1, the Atangana-Baleanu-Caputo fractional derivative of order α is defined as:

​ABCDtαft=Bα1-α0tEα-αt-τα1-αf'τdτ,

where Bα is a normalization function with B0=B1=1, and Eα⋅ is the Mittag-Leffler function.

Definition 2.2: (Mittag-Leffler Function).[7]  The one-parameter Mittag-Leffler function is defined as: Eαz=k=0∞zkΓαk+1, α>0,z∈C.

The two-parameter Mittag-Leffler function is:

Eα,βz=k=0∞zkΓαk+β, α,β>0,z∈C.

2.2 Mittag-Leffler Stability Theory

Definition 2.3 (Mittag-Leffler Stability) [7].  The solution xt of a fractional-order system is said to be Mittag-Leffler stable if: ∥xt∥≤mx0Eα-λtαb where x0 is the initial condition, m0=0, mx≥0, λ>0, and b>0 are constants.

Remark 2.1.  When α=1, Mittag-Leffler stability reduces to exponential stability since E1-λt=e-λt. For 0<α<1, the decay is typically slower than exponential, reflecting the memory effects in fractional systems.

Lemma 2.1 (Fractional Comparison Principle). [11] Let ut∈C10,T,R+ satisfy: ​ABCDtαut≤-λut, u0=u0 Then: ut≤u0Eα-λtα

3. Model Formulation

Consider the fractional-order system with Atangana-Baleanu-Caputo derivatives:

tABCC= aN - LCSP- BCW - ΦCC

tABCSp= LCSp - δ1 Sp W - δ2 Sp F - Φ2 Sp - γ1 Sp

tABCW= δ1 Sp W - ρ WF - Φ3 W

tABCF= δ2 Sp F + ρ WF - γ2 F - Φ4 F

tABCSR= γ1 Sp + γ2F - Φ5 SR

(1.1)

With the initial conditions

C0=C0≥0, Sp0=Sp0≥0, W0=W0≥0, F0=F0≥0, SR=SR0≥0.

a – Effluent Concentration

N – Total Area

L – Rate of contaminated sediments in soil

β – Rate of contaminated settlements in water

δ1 – Washout rate of effluents from soil into water

δ2 – Polluted settlements in farmland

ρ – Polluted water affecting farmland

γ1 – Remedy in polluted soil

γ2- remedial measure in farmland

ϕ1, ϕ2,ϕ3, ϕ4, ϕ5 - Mortality rate of all the compartments

C- Effluent Concentration

Sp – Polluted Soil

W – Water Contamination

F – Farmland pollution

SR – Remedied Soil

Definition 3.4. [7]  The system is said to be locally asymptotically stable at an equilibrium point E* if all eigenvalues λ of the Jacobian matrix satisfy argλ>απ2.

Definition 3.5.[7]  The system is globally asymptotically stable at an equilibrium point E* if it is stable and all trajectories approach E* as t→∞.

4. Equilibrium Point and Stability analysis:

The equilibrium points for the fractional-order pollution remediation model are found by setting the right-hand sides of the model equations (1.1) to zero.

Trivial Equilibrium Point (E0):

This equilibrium represents a state where pollution is absent in the soil, water, and farmland compartments.

E0 = aN∅C , 0, 0, 0, 0.

The components are:

C0 =aN∅C : Effluent concentration reaches a baseline level.

SP0= 0: No polluted soil.

W0 = 0: No water contamination.

F0 = 0: No farmland pollution.

SR0= 0: No remediated soil.

Endemic Equilibrium Point (E*):

This equilibrium represents a state where pollution persists in all environmental compartments.

E* = C*, Sp*, W*, F*, SR*.

where C* > 0, Sp* > 0, W* > 0, F* > 0, and SR* > 0.The specific values are determined by solving the system of algebraic equations and we obtained,

C* =aN - LCS - BCWΦC

Sp* = LCSδ1 W + δ2 F + Φ2 + γ1

W* = δ1 Sp(ρ F + Φ3)

F* =δ2 Sp + ρ Wγ2 + Φ4

SR* =γ1 Sp + γ2 FΦ5.

4.2 Stability analysis

In this section, we analyze the stability properties of the fractal-fractional order model for co-contaminated soil and water pollution. The system is governed by the Atangana-Baleanu-Caputo (ABC) fractional derivative, which enables the incorporation of memory effects and non-local dynamics into the pollution transmission process.

4.2.1 Local Stability Analysis

Jacobian Matrix Computation

Let X=C,Sp,W,F,SRT. The Jacobian matrix JE evaluated at equilibrium point E=C*,Sp*,W*,F*,SR* is:

JE=-ΦC-LSp*-BW* -LC* -BC* 0 0 LSp* LC*-δ1W*-δ2F*-Φ2-γ1 -δ1Sp* -δ2Sp* 0 0 δ1W* δ1Sp*-ρF*-Φ3 -ρW* 0 0 δ2F* ρF* δ2Sp*+ρW*-γ2-Φ4 0 0 γ1 0 γ2 -Φ5

Local Stability of Trivial Equilibrium

Proposition 4.1.  The trivial equilibrium point E0=aNΦC,0,0,0,0 has Jacobian matrix:

JE0=-ΦC -LaNΦC -BaNΦC 0 0 0 LaNΦC-Φ2-γ1 0 0 0 0 0 -Φ3 0 0 0 0 0 -γ2-Φ4 0 0 γ1 0 γ2 -Φ5

Theorem 4.1.  The trivial equilibrium E0 is locally asymptotically stable if: LaNΦC<Φ2+γ1

Proof. The characteristic equation is:

detJE0-λI=-ΦC-λ-Φ3-λ-Φ5-λ-γ2-Φ4-λLaNΦC-Φ2-γ1-λ=0

The eigenvalues are:

- λ1=-ΦC<0

- λ2=-Φ3<0

- λ3=-Φ5<0

- λ4=-γ2+Φ4<0

- λ5=LaNΦC-Φ2-γ1

For stability, we require λ5<0, which gives the condition:

LaNΦC<Φ2+γ1

Since 0<α<1, we have απ2<π2, so cosαπ2>0. For real eigenvalues, the stability condition argλ>απ2 is equivalent to λ<0 when λ is real and negative. ◻

Local Stability of Endemic Equilibrium

Theorem 4.2 (Local Stability of Endemic Equilibrium).  The endemic equilibrium point E* is locally asymptotically stable if all eigenvalues λ of the Jacobian matrix JE* satisfy argλ>απ2 where 0<α<1 is the fractional order.

Proof. The characteristic equation is detJE*-λI=0. Due to the triangular structure of the last row, one eigenvalue is λ5=-Φ5<0, which is always stable.

For the remaining 4×4 subsystem, let:

J4=-ΦC-LSp*-BW* -LC* -BC* 0 LSp* 0 -δ1Sp* -δ2Sp* 0 δ1W* 0 -ρW* 0 δ2F* ρF* 0

The characteristic polynomial of J4 is:

P4λ=λ4+a3λ3+a2λ2+a1λ+a0

where: To derive the coefficients of the characteristic polynomial, we compute the determinant of J4-λI:

J4-λI=-ΦC-LSp*-BW*-λ -LC* -BC* 0 LSp* -λ -δ1Sp* -δ2Sp* 0 δ1W* -λ -ρW* 0 δ2F* ρF* -λ

Expanding along the first row:

detJ4-λI=-ΦC-LSp*-BW*-λM11+LC*M12+BC*M13

where Mij are the corresponding cofactors.

Computing the cofactors:

M11=det-λ -δ1Sp* -δ2Sp* δ1W* -λ -ρW* δ2F* ρF* -λ

Expanding this 3×3 determinant:

M11 =-λλ2+ρW*⋅ρF*--ρW*δ2F*   +δ1Sp*δ1W*⋅-λ--ρW*δ2F*   -δ2Sp*δ1W*⋅ρF*--λδ2F*  =-λ3-λρ2W*F*-λδ12Sp*W*-λδ22Sp*F*   +δ1δ2ρSp*W*F*+δ1δ2ρSp*W*F*  =-λ3-λρ2W*F*+δ12Sp*W*+δ22Sp*F*+2δ1δ2ρSp*W*F*

M12=-detLSp* -δ1Sp* -δ2Sp* 0 -λ -ρW* 0 ρF* -λ

M12=-LSp*λ2+ρ2W*F*=-LSp*λ2-LSp*ρ2W*F*

M13=detLSp* -λ -δ2Sp* 0 δ1W* -ρW* 0 δ2F* ρF*

M13=LSp*δ1W*⋅ρF*--ρW*δ2F*=LSp*ρδ1W*F*+δ2W*F*

Combining all terms:

detJ4-λI =-ΦC-LSp*-BW*-λ[-λ3-λρ2W*F*+δ12Sp*W*+δ22Sp*F*   +2δ1δ2ρSp*W*F*]   +LC*-LSp*λ2-LSp*ρ2W*F*   +BC*LSp*ρδ1+δ2W*F*

Expanding and collecting terms by powers of λ:

detJ4-λI=λ4+a3λ3+a2λ2+a1λ+a0

where the coefficients are:

a3 =ΦC+LSp*+BW* a2 =ρ2W*F*+δ12Sp*W*+δ22Sp*F*+LC*⋅LSp*   -LC*⋅δ1W*+BC*⋅ρδ1+δ2F* a1 =-2δ1δ2ρSp*W*F*ΦC+LSp*+BW*   -LC*LSp*ρ2W*F*-LC*LSp*δ12W*-LC*LSp*δ22F*   +BC*LSp*ρ2δ1+δ2W*F* a0 =2δ1δ2ρSp*W*F*ΦC+LSp*+BW*   +LC*LSp*ρ2W*F*+BC*LSp*ρ2δ1+δ2W*F*

For the determinant of the full Jacobian matrix:

detJE*-λI=λ+Φ5⋅detJ4-λI

Therefore, the constant term of the full characteristic polynomial is:

a0full=Φ5⋅a0

By the Routh-Hurwitz criterion adapted for fractional systems, E* is locally asymptotically stable if:

- a3>0

- a3a2>a1

- a3a2a1>a12+a32a0

- a0>0

- a3a2a1a0>a12a0+a32a02

Since all system parameters are positive and at endemic equilibrium Sp*,W*,F*>0, conditions (1) and (4) are automatically satisfied. The remaining conditions depend on the specific parameter values and equilibrium magnitudes. ◻

Corollary 4.1.  If the endemic equilibrium exists (R0>1) and the trace of J4 is negative with positive determinant, then the endemic equilibrium is locally asymptotically stable for sufficiently small fractional order α.

4.3 Global Stability Analysis

Global Stability of Trivial Equilibrium

Theorem 4.3.  The trivial equilibrium E0 is globally asymptotically stable in R+5 if: LaNΦC<Φ2+γ1

Proof. We construct a Lyapunov function for the subsystem involving Sp,W,F:

V1Sp,W,F=Sp+W+F

The fractional derivative of V1 along solutions is:

​ABCDtαV1 =ABCDtαSp+ABCDtαW+ABCDtαF  =LCSp-δ1SpW-δ2SpF-Φ2Sp-γ1Sp   +δ1SpW-ρWF-Φ3W   +δ2SpF+ρWF-γ2F-Φ4F  =LCSp-Φ2Sp-γ1Sp-Φ3W-γ2F-Φ4F  =SpLC-Φ2-γ1-Φ3W-γ2+Φ4F

If LC<Φ2+γ1, then ​ABCDtαV1≤0 with equality if and only if Sp=W=F=0.

For the full system, consider:

VC,Sp,W,F,SR=C-C0-C0lnCC0+Sp+W+F+SR-SR0-SR0lnSRSR0

where C0=aNΦC and SR0=0.

The derivative of the first term is:

​ABCDtαC-C0-C0lnCC0 =1-C0CABCDtαC  =1-C0CaN-LCSp-BCW-ΦCC  =1-C0C-LCSp-BCW  =-LCSp1-C0C-BCW1-C0C

Since C0=aNΦC and at equilibrium aN=ΦCC0:

​ABCDtαC-C0-C0lnCC0=-LCSp1-C0C-BCW1-C0C≤0

For the remediated soil component:

​ABCDtαSR=γ1Sp+γ2F-Φ5SR

Combining all terms and using LaSalle’s invariance principle for fractional systems, we conclude global asymptotic stability when LaNΦC<Φ2+γ1. ◻

Global Stability of Endemic Equilibrium

Theorem 4.4.  If the endemic equilibrium E* exists and the following conditions hold:

- LaNΦC>Φ2+γ1 (existence condition)

- Additional technical conditions on the parameters

then E* is globally asymptotically stable in the interior of R+5.

Proof. We construct a Lyapunov function of Goh-Volterra type:

V=CHCC+SpHSpSp+WHWW+FHFF+SRHSRSR

where Hx=x-1-lnx and E=C,Sp,W,F,SR is the endemic equilibrium.

Using the equilibrium equations and following the approach in , we can show that ​ABCDtαV≤0 with equality if and only if C,Sp,W,F,SR=C,Sp,W,F,SR.

The detailed computation involves substituting the system equations and equilibrium relations, and requires that all cross terms cancel appropriately, leading to a negative definite derivative. ◻

4.3 Basic Reproduction Number and Threshold Dynamics

Definition 4.6.[5]   The basic reproduction number R0 is defined as the expected number of secondary pollution cases produced by a single primary case in a completely susceptible environment.

Theorem 4.5.  For the pollution model, the basic reproduction number is: R0=L⋅aNΦCΦ2+γ1

Proof. Using the next-generation matrix method, we identify:

- New pollution generation terms: LCSp

- Transfer terms: Φ2Sp+γ1Sp

At the trivial equilibrium E0, we have C0=aNΦC, so:

R0=L⋅aNΦCΦ2+γ1=L⋅aNΦCΦ2+γ1

Corollary 2.  The stability threshold can be expressed in terms of the basic reproduction number:

- If R0<1, then E0 is globally asymptotically stable

- If R0>1, then E0 is unstable and the endemic equilibrium exists

4.4 Mittag-Leffler Stability Analysis

Fractional Lyapunov Direct Method

Theorem 4.6 (Fractional Lyapunov Stability Theorem). Let D⊂Rn be a domain containing the origin. If there exists a continuously differentiable function Vt,x:[0,∞)×D→R and class K functions α1,α2,α3 such that:

α1∥x∥ ≤Vt,x≤α2∥x∥ ​ABCDtαVt,x ≤-α3∥x∥

then the origin is asymptotically Mittag-Leffler stable.

Mittag-Leffler Stability of Trivial Equilibrium

Theorem 4.7 (Main Result - Mittag-Leffler Stability of E0).  The trivial equilibrium point E0=aNΦC,0,0,0,0 is Mittag-Leffler stable if: R0=L⋅aNΦCΦ2+γ1<1

Proof. We construct a fractional Lyapunov function candidate:

Vt,X=V1t,C+V2t,Sp+V3t,W+V4t,F+V5t,SR

where:

V1t,C =12C-C02 V2t,Sp =12Sp2 V3t,W =12W2 V4t,F =12F2 V5t,SR =12SR2

Bounding the Lyapunov Function, Clearly:

12∥X-E0∥2≤Vt,X≤12∥X-E0∥2

where ∥X-E0∥2=C-C02+Sp2+W2+F2+SR2.

Computing the Fractional Derivative, using the property of fractional derivatives:

​ABCDtαVt,X=i=15∂V∂xi⋅ABCDtαxi

Computing each term:

For V1:

​ABCDtαV1=C-C0⋅ABCDtαC=C-C0aN-LCSp-BCW-ΦCC

Since at equilibrium aN=ΦCC0:

​ABCDtαV1=C-C0-LCSp-BCW-ΦCC-C0

=-C-C02ΦC-C-C0LCSp-C-C0BCW

For V2:

​ABCDtαV2=Sp⋅ABCDtαSp=SpLCSp-δ1SpW-δ2SpF-Φ2Sp-γ1Sp

=Sp2LC-δ1W-δ2F-Φ2-γ1-δ1Sp2W-δ2Sp2F

For V3:

​ABCDtαV3=W⋅ABCDtαW=Wδ1SpW-ρWF-Φ3W

=δ1SpW2-ρW2F-Φ3W2

For V4:

​ABCDtαV4=F⋅ABCDtαF=Fδ2SpF+ρWF-γ2F-Φ4F

=δ2SpF2+ρWF2-γ2+Φ4F2

For V5:

​ABCDtαV5=SR⋅ABCDtαSR=SRγ1Sp+γ2F-Φ5SR

=γ1SpSR+γ2FSR-Φ5SR2

Summing all terms:

​ABCDtαVt,X =-C-C02ΦC-C-C0LCSp-C-C0BCW   +Sp2LC-δ1W-δ2F-Φ2-γ1-δ1Sp2W-δ2Sp2F   +δ1SpW2-ρW2F-Φ3W2   +δ2SpF2+ρWF2-γ2+Φ4F2   +γ1SpSR+γ2FSR-Φ5SR2

Using Young’s inequality and Cauchy-Schwarz inequality to bound cross terms:

For C-C0LCSp:

C-C0LCSp≤ϵ12C-C02+L2C022ϵ1Sp2

For C-C0BCW:

C-C0BCW≤ϵ22C-C02+B2C022ϵ2W2

For γ1SpSR:

γ1SpSR≤ϵ32Sp2+γ122ϵ3SR2

For γ2FSR:

γ2FSR≤ϵ42F2+γ222ϵ4SR2

Choosing appropriate ϵi values and collecting terms:

​ABCDtαVt,X≤-λ1C-C02-λ2Sp2-λ3W2-λ4F2-λ5SR2

where the coefficients depend on system parameters and the condition R0<1.

Specifically, for the Sp term:

λ2=Φ2+γ1-LCC0-(positive correction terms)

Since C0=aNΦC:

λ2=Φ2+γ1-LaNΦC=Φ2+γ11-R0-(positive terms)

Therefore, if R0<1, then λ2>0 for sufficiently small cross-term contributions.

We have established:

​ABCDtαVt,X≤-λVt,X

where λ=min{λ1,λ2,λ3,λ4,λ5}>0 when R0<1.

By the fractional comparison principle:

Vt,X≤V0,X0Eα-λtα

Since Vt,X≥12∥X-E0∥2:

∥Xt-E0∥2≤2V0,X0Eα-λtα

Therefore:

∥Xt-E0∥≤2V0,X0Eα-λtα1/2

This exact proportionality allows the decay of the Lyapunov function to be directly translated into the decay of the state variables, which plays a crucial role in proving Mittag–Leffler stability for the considered fractional-order model. ◻

Convergence Rate Analysis

Corollary 4.3 (Convergence Rate).  Under the conditions of Theorem 1, the solution converges to the trivial equilibrium with rate: ∥Xt-E0∥=OEα-λtα

For long times, the Mittag-Leffler function behaves as: Eα-λtα∼1Γ1-α1λtα, t→∞

This shows algebraic decay, which is slower than exponential decay for α<1.

Parameter Sensitivity Analysis

The Mittag-Leffler stability condition R0<1 can be written as:

L<ΦCΦ2+γ1aN

This provides clear guidelines for pollution control:

- Reduce pollution generation rate L

- Increase degradation rates ΦC,Φ2,γ1

- Reduce system size N or effluent rate a

Mittag-Leffler Stability of Endemic Equilibrium

Theorem 4.8 (Mittag-Leffler Stability of Endemic Equilibrium).  If the endemic equilibrium E* exists (i.e., R0>1) and certain technical conditions are satisfied, then E* is locally Mittag-Leffler stable.

Proof. The proof follows similar steps using a Lyapunov function of the form:

Vt,X=i=15xiHxixi

where Hx=x-1-lnx is the Volterra function, and X=C,Sp,W,F,SR is the endemic equilibrium.

The fractional derivative satisfies:

​ABCDtαVt,X≤-μi=15xi-xi2

for some μ>0, establishing local Mittag-Leffler stability. ◻

5. Numerical Scheme for ABC Fractional Model

Trapezoidal Product-Integration Rule for ABC Derivatives

We employ the advanced trapezoidal product-integration scheme recently developed for Atangana-Baleanu-Caputo fractional derivatives, which provides higher accuracy and better stability properties.

The numerical approximation at time tn=nΔt is given by:

​ABCDtαutn≈Bα1-αk=0nwn-kαutk-ut0+αΔtαΓ2-α1-αk=0nwn-kαutk-utk-1Δt

where the weights are defined as:

w0α =1 wkα =k+11-α-2k1-α+k-11-α, k≥1 w0α =0 wkα =k+12-α-2k2-α+k-12-α, k≥1

Improved Predictor-Corrector Scheme

We implement an improved second-order predictor-corrector method with adaptive step-size control:

Predictor Step:

Xn+1P=X0+1-αBαFtn,Xn+αBαΓαj=0nbj,n+1Ftj,Xj

Corrector Step:

Xn+1=X0+1-αBαFtn+1,Xn+1P+αBαΓαj=0n+1bj,n+1Ftj,Xjc

where:

bj,n+1 ={Δtααn+1α-nα, j=0 Δtααn-j+2α+1-2n-j+1α+1+n-jα+1, 1≤j≤n Δtαα, j=n+1

bj,n+1 ={Δtααα+1n+1α+1-n+1-αn+1α, j=0 Δtααn-j+2α+1-2n-j+1α+1+n-jα+1, 1≤j≤n Δtααα+1n+1α+1-n+1-αn+1α, j=n+1

Implementation for the Pollution Model

For our specific system, let Xt=Ct,Spt,Wt,Ft,SRtT and FX=F1X,F2X,F3X,F4X,F5XT where:

F1X =aN-LCSp-BCW-ΦCC F2X =LCSp-δ1SpW-δ2SpF-Φ2Sp-γ1Sp F3X =δ1SpW-ρWF-Φ3W F4X =δ2SpF+ρWF-γ2F-Φ4F F5X =γ1Sp+γ2F-Φ5SR

The numerical scheme becomes:

Component-wise Predictor:

Cn+1P =C0+1-αBαF1tn,Xn+αBαΓαj=0nbj,n+1F1tj,Xj Sp,n+1P =Sp,0+1-αBαF2tn,Xn+αBαΓαj=0nbj,n+1F2tj,Xj Wn+1P =W0+1-αBαF3tn,Xn+αBαΓαj=0nbj,n+1F3tj,Xj Fn+1P =F0+1-αBαF4tn,Xn+αBαΓαj=0nbj,n+1F4tj,Xj SR,n+1P =SR,0+1-αBαF5tn,Xn+αBαΓαj=0nbj,n+1F5tj,Xj

Component-wise Corrector:

Cn+1 =C0+1-αBαF1tn+1,Xn+1P+αBαΓαj=0n+1bj,n+1F1tj,Xjc Sp,n+1 =Sp,0+1-αBαF2tn+1,Xn+1P+αBαΓαj=0n+1bj,n+1F2tj,Xjc Wn+1 =W0+1-αBαF3tn+1,Xn+1P+αBαΓαj=0n+1bj,n+1F3tj,Xjc Fn+1 =F0+1-αBαF4tn+1,Xn+1P+αBαΓαj=0n+1bj,n+1F4tj,Xjc SR,n+1 =SR,0+1-αBαF5tn+1,Xn+1P+αBαΓαj=0n+1bj,n+1F5tj,Xjc

5. Numerical Simulations and Discussion

To validate the analytical stability results derived in Sections local stability and global stability, and to illustrate the model’s behavior under the condition R0>1, we conducted numerical simulations using the advanced predictor-corrector scheme. For this demonstration, system parameters were selected such that the basic reproduction number is R0=4.55.

Figure 1 presents the time-series dynamics of all five state variables for R0=4.55. The simulation confirms the theoretical predictions. Initially polluted compartments (Spt, Wt, Ft) do not decay to zero. Instead, all variables approach distinct, positive steady-state values, indicating convergence to the stable endemic equilibrium E*. This behavior is consistent with the model’s prediction of persistent pollution when R0>1.

Fig1: Simulated time evolution of all state variables for the fractional-order pollution model with α=0.95 and R0=4.55. Initial conditions were C0,Sp0,W0,F0,SR0=30,2,1,0.5,0. The graph shows that all variables approach positive steady-state values (C*,Sp*,W*,F*,SR*), confirming the theoretical prediction of global asymptotic stability of the endemic equilibrium E* for R0>1.

To further analyze the system’s behavior, we examine the dynamics of each compartment individually.

Fig2: Simulated dynamics of Effluent Concentration Ct for R0=4.55. The concentration approaches a positive steady-state value C*, distinct from the level C0=aN/ΦC associated with the unstable pollution-free equilibrium E0.

Fig3: Simulated dynamics of Polluted Soil Spt for R0=4.55. The level increases from its initial value and stabilizes at a positive value Sp*. This confirms the existence and stability of the endemic equilibrium E*, as Sp*>0 is a defining characteristic.

Fig4: Simulated dynamics of Water Contamination Wt for R0=4.55. The contamination level rises and approaches a positive steady state W*, indicating that water pollution is sustained rather than eliminated.

Fig5: Simulated dynamics of Farmland Pollution Ft for R0=4.55. The pollution level converges to a positive equilibrium value F*, verifying that E* is the long-term attractor, with F*>0, signifying persistent contamination of agricultural land.

Fig 6: Simulated dynamics of Remedied Soil SRt for R0=4.55. The amount of remediated soil accumulates over time and reaches a steady state SR*. This reflects the balance between ongoing remediation processes (γ1Sp*, γ2F*) and the degradation rate (Φ5SR*) at the stable equilibrium E*.

The numerical simulations for R0=4.55 provide strong evidence supporting the theoretical stability analysis:

- Effluent Concentration Ct (Figure 2): The effluent concentration Ct does not settle at the level C0=aN/ΦC associated with E0. Instead, it approaches a different positive steady state C*. This deviation from C0 is expected at E* due to the active interaction terms (-LC*Sp*, -BC*W*) which are zero at E0 but non-zero at E*.

- Polluted Soil Spt (Figure 3): The level of soil pollution increases from its initial value and stabilizes at a positive value Sp*. This is a direct confirmation of the existence and stability of E*, as Sp*>0 is a defining characteristic of the endemic equilibrium. The convergence to Sp* demonstrates that soil pollution becomes persistent in the system when R0>1.

- Water Contamination Wt (Figure 4): Similarly, the water contamination level Wt rises and approaches a positive steady state W*. This behavior aligns with the stability of E*, where W*>0. It indicates that water pollution is sustained rather than eliminated.

- Farmland Pollution Ft (Figure 5): The farmland pollution level Ft also converges to a positive equilibrium value F*. This verifies that E* is the long-term attractor, with F*>0, signifying persistent contamination of agricultural land.

- Remedied Soil SRt (Figure 6): The amount of remediated soil SRt accumulates over time and reaches a steady state SR*. This reflects the ongoing remediation processes (γ1Sp*, γ2F*) balancing the degradation rate (Φ5SR*) at the stable equilibrium E*. The fact that it reaches a finite value SR*=γ1Sp*+γ2F*/Φ5 shows that remediation reaches a balance with persistent pollution sources, rather than ceasing.

In summary, the numerical simulations for R0=4.55 provide strong evidence supporting the theoretical stability analysis. The system’s trajectory converges to the unique endemic equilibrium E*, characterized by persistent, non-zero levels of pollution across all active compartments (Sp*,W*,F*), a corresponding effluent level C*, and a balanced remediation state SR*. This confirms that when R0>1, the pollution-free state E0 is unstable, and the system settles into a persistent pollution regime as predicted by the model.

6. Conclusion

This study presented a comprehensive analysis of a five-compartment fractional-order model for co-contaminated soil and water pollution dynamics using the Atangana-Baleanu-Caputo derivative. We derived the basic reproduction number R0 as the fundamental threshold parameter governing system behavior:

- When R0< 1, the pollution-free equilibrium is globally asymptotically stable and Mittag-Leffler stable, indicating successful pollution elimination

- When R0> 1, a unique endemic equilibrium exists and is globally asymptotically stable, predicting persistent pollution across all environmental compartments

As predicted, the pollution-free equilibrium E0 is unstable. Initially polluted compartments (Spt, Wt, Ft) approach distinct positive steady-state values (Sp*, W*, F*), indicating persistent pollution. The effluent concentration Ct stabilizes at C*, which differs from the level C0=aN/ΦC associated with E0. The remediated soil SRt accumulates and reaches a steady state SR*, reflecting a balance between ongoing remediation (γ1Sp*, γ2F*) and degradation (Φ5SR*). This behavior validates the model’s prediction of a persistent pollution regime when R0>1. The system’s trajectory converges to E*, characterized by non-zero pollution levels across all active compartments. These results confirm that R0=4.55 leads to a stable, persistent pollution state rather than elimination.

Our analysis employed advanced techniques including fractional Lyapunov functionals and Mittag-Leffler stability theory to characterize long-term system behavior precisely. We developed an adaptive predictor-corrector numerical scheme specifically tailored for ABC fractional systems, enabling accurate long-term simulations.

This work provides crucial analytical and computational tools for understanding complex pollution dynamics in interconnected environmental media. The threshold condition R0< 1 offers practical guidance for pollution control strategies: reducing pollutant input rates, enhancing degradation/remediation processes, and limiting transmission between compartments. Future work will explore optimal control strategies for driving R0 below the critical threshold and investigate spatially explicit extensions of the model.

CRediT authorship contribution statement:

Priya P. and Roselyn Besi P. Conceptualization, Methodology, Writing-Original draft, formal analysis, investigation.

Sabarmathi A.: Scrutinized the work.

Declaration of Competing interest:

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

Data Availability:

Data will be provided by corresponding author on reasonable request.

Funding:

No Funding

References

- Atangana, A., & Baleanu, D. (2017). New fractional derivatives with nonlocal and non-singular kernel: Theory and application to heat transfer model. Thermal Science, 21(1), 1-7.

- Area, I., Batarfi, H., Losada, J., Nieto, J. J., Shammakh, W., & Torres, A. (2015). On a fractional order Ebola epidemic model. Advances in Difference Equations, 2015(1), 278.

- Diethelm, K. (2010). The analysis of fractional differential equations: An application-oriented exposition using differential operators of Caputo type. Springer Science & Business Media.

- Deeksha Singh, Farheen Sultana, Rajesh K. Pandey, “Approximation of Caputo-Prabhakar derivative with application in solving time fractional advection-diffusion equation”, 2022,  https://doi.org/10.1002/fld.5077.

- Swati Yadav, Rajesh K. Pandey, Anil K. Shukla,” Numerical approximations of Atangana–Baleanu Caputo derivative and its application”, Chaos, Solitons & Fractals, Volume 118, 2019, Pages 58-64, ISSN 0960-0779, https://doi.org/10.1016/j.chaos.2018.11.009.

- Baleanu, D., Diethelm, K., Scalas, E., & Trujillo, J. J. (2019). Fractional calculus: models and numerical methods (Vol. 5). World Scientific.

- Li, C., & Zhang, F. (2019). A survey on the stability of fractional differential equations. The European Physical Journal Special Topics, 228(1), 73-84.

- Vargas-De-León, C. (2012). Volterra-type Lyapunov functions for fractional-order epidemic systems. Communications in Nonlinear Science and Numerical Simulation, 17(9), 3617-3622.

- Baleanu, D., Machado, J. A. T., & Luo, A. C. J. (Eds.). (2012). Fractional dynamics and control. Springer Science & Business Media.

- Pagnini, G. (2019). Erdélyi-Kober fractional diffusion. In Handbook of Fractional Calculus with Applications (pp. 265-280). De Gruyter.

- Sheikh, N. A., Dennis, L. C. C., Ijarola, O. A., Zainal, A. A., & Mohamad, M. (2022). A review of the developments in the field of fractional calculus and its applications. Alexandria Engineering Journal, 61(12), 10137-10153.

- Singh, D., Sultana, F., Pandey, R.K. et al. A comparative study of three numerical schemes for solving Atangana–Baleanu fractional integro-differential equation defined in Caputo sense. Engineering with Computers 38 (Suppl 1), 149–168 (2022).

- Atangana, A., & Algahtani, O. J. J. (2015). Extension of the groundwater flow model to the concept of fractional variable order derivative. Abstract and Applied Analysis, 2015.

- Sun, H., Chang, A., Zhang, Y., & Chen, W. (2019). Modeling anomalous heat transport in geothermal reservoirs via fractional Laplacian. International Journal of Numerical and Analytical Methods in Geomechanics, 43(14), 2327-2342.

- Sardanyes, J., Farner, S., & Alsedà, L. (2022). Bifurcations and long-term dynamics in a discrete ecological model with fractional-order recurrences. Chaos, Solitons & Fractals, 154, 111654.

- Swati Yadav, Rajesh K. Pandey,” Numerical approximation of fractional burgers equation with Atangana–Baleanu derivative in Caputo sense”, Chaos, Solitons & Fractals, Volume 133, 2020, 109630, ISSN 0960-0779.